August 2026

|  | Florida ITS Architecture Support and Maintenance Project District 5 Conversion Report (ARC-IT Version 9.4) |
| --- | --- |

Document Version Control

| Author / Action | Submittal Date | Version No. |
| --- | --- | --- |
| David Binkley / Draft Document Development | August 17, 2026 | A9.4 08/2026 |
| Cliff Heise/ QA/QC Review | August 17, 2026 | A9.4 08/2026 |

# Table of Contents

1.0	Introduction	4

2.0	Description of Changes	4

3.0	Architecture Conversion Results	5

3.1	Architecture Inventory Elements	5

3.2	Architecture Information Flows	5

3.3	Architecture Functional Requirements	5

# List of Tables

Table 1. Conversion Analysis of Functional Requirements	6

# Introduction

This Architecture Conversion Report records the Florida District 5 Regional ITS Architecture (RITSA) update from its reference in the Architecture Reference for Cooperative and Intelligent Transportation (ARC-IT) Version 9.3 to ARC-IT Version 9.4. This report addresses notable results from the conversion process.

ARC-IT is the national architecture reference for ITS. Version 9.4, per the https://www.arc-it.net/ website, includes new services for vehicle automation, maritime port information systems, and support for new vehicle-to-everything (V2X) communications methods; including networked V2X, wide area wireless (cellular), and introducing the use of multi-access edge computing (MEC).

# Description of Changes

The architecture conversion process uses the Regional Architecture Development for Intelligent Transportation (RAD-IT) software Version 9.4 to convert the architecture to be compatible with ARC-IT Version 9.4. The process includes the following steps to accomplish the conversion.

- Architecture conversion: Conversion features in RAD-IT Version 9.4 convert the architecture database schema to be compatible with RAD-IT Version 9.4 and aligned to reference ARC-IT Version 9.4 content.

- Conversion analysis: Conversion information is produced by RAD-IT for the architecture conversion noting the changes made. The conversion information notes the schema and content changes, such as service splits or consolidations, element mapping changes, information flow changes, and communications standards changes. Analysis is required for each converted item to assess the appropriateness of each change for the architecture.

- Architecture content update: The intent of the conversion process was to maintain the alignment of the converted Architecture content to the greatest extent possible with the pre-conversion Architecture content. Element physical object mapping changes, service package changes, information flow additions and adjustments, and the evolution of the standards mappings in ARC-IT Version 9.4 required changes to be made to the Architecture content. Unless it was necessary, no additional changes beyond those required to align the pre-conversion and converted architecture content were made. During the course of the Annual Architecture Maintenance Update, ARC-IT Version 9.4 features that could be considered as additional information to the Architecture will be assessed.

- Architecture website posting: The converted architecture will be posted to the Florida ITS Architecture website.

# Architecture Conversion Results

The District 5 RITSA was converted to be compatible with ARC-IT Version 9.4. The following sections highlight the changes made to the architecture as a result of the conversion process.

## Architecture Inventory Elements

For the conversion from 9.3 to 9.4 there were no changes to the architecture inventory of elements.

## Architecture Information Flows

No architecture information flows were impacted by the conversion process.

## Architecture Functional Requirements

Table 2 provides conversion results for architecture functional requirements impacted by the conversion process. The table shows the element impacted, the type of change made, the old functional object, number, and requirement, along with the new functional object, number, and requirement to display the change made.

Table 1. Conversion Analysis of Functional Requirements

| Element Name | Change | Old Functional Object | Old Num | Old Req | New Functional Object | New Num | New Req |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Brevard County Field Equipment | Replaced | Roadway Basic Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. | Roadway Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. |
| Brevard County Field Equipment | Replaced | Roadway Basic Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. | Roadway Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. |
| Brevard County Field Equipment | Replaced | Roadway Basic Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. | Roadway Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. |
| CFX CAV Field Equipment | Modified | RSE Map Management | 12 | The field element shall provide roadway geometry updates to proximate Vehicles. | RSE Map Management | 12 | The field element shall acquire roadway geometry information from mapping centers. |
| CFX CAV Field Equipment | Modified | RSE Map Management | 9 | The field element shall provide basemap updates to proximate Vehicles. | RSE Map Management | 9 | The field element shall acquire basemap updates from mapping centers. |
| CFX CAV Field Equipment | Replaced | Roadway Basic Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. | Roadway Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. |
| CFX CAV Field Equipment | Replaced | Roadway Basic Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. | Roadway Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. |
| CFX Field Equipment | Replaced | Roadway Basic Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. | Roadway Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. |
| CFX Field Equipment | Replaced | Roadway Basic Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. | Roadway Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. |
| CFX Field Equipment | Replaced | Roadway Basic Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. | Roadway Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. |
| CFX Field Equipment | Replaced | Roadway Basic Surveillance | 5 | The field element shall return sensor and CCTV system fault data to the controlling center for repair. | Roadway Surveillance | 5 | The field element shall return sensor and CCTV system fault data to the controlling center for repair. |
| CFX Field Equipment | Replaced | Roadway Basic Surveillance | 6 | The field element shall collect, process, and send traffic images to the center for further wrong way vehicle entries analysis and detection. | Roadway Surveillance | 6 | The field element shall collect, process, and send traffic images to the center for further wrong way vehicle entries analysis and detection. |
| City of Altamonte Springs AV Shuttle | Modified | Vehicle Control Automation | 3 | The vehicle shall evaluate the likelihood of a collision between two vehicles or a vehicle and a stationary object, based on the proximity of other objects to the vehicle and the current speed and direction of the vehicle. | Vehicle Control Automation | 3 | The vehicle shall evaluate the likelihood of a collision between two vehicles or a vehicle and another objects, based on the proximity and vectors of other objects relative to the vehicle and the current speed and direction of the vehicle. |
| City of Altamonte Springs AV Shuttle | Modified | Vehicle Control Warning | 2 | The vehicle shall evaluate the likelihood of a collision between two vehicles or a vehicle and a stationary object to warn the driver, based on the proximity of other objects to the vehicle, roadway characteristics, and the current speed and direction of the vehicle. | Vehicle Control Warning | 2 | The vehicle shall evaluate the likelihood of a collision between two vehicles or a vehicle and another object to warn the driver, based on the proximity and vectors of other objects to the vehicle, roadway characteristics, and the current speed and direction of the vehicle. |
| City of Daytona Beach Field Equipment | Replaced | Roadway Basic Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. | Roadway Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. |
| City of Daytona Beach Field Equipment | Replaced | Roadway Basic Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. | Roadway Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. |
| City of Daytona Beach Field Equipment | Replaced | Roadway Basic Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. | Roadway Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. |
| City of Maitland Field Equipment | Replaced | Roadway Basic Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. | Roadway Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. |
| City of Maitland Field Equipment | Replaced | Roadway Basic Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. | Roadway Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. |
| City of Melbourne Field Equipment | Replaced | Roadway Basic Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. | Roadway Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. |
| City of Melbourne Field Equipment | Replaced | Roadway Basic Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. | Roadway Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. |
| City of Melbourne Field Equipment | Replaced | Roadway Basic Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. | Roadway Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. |
| City of Melbourne Field Equipment | Replaced | Roadway Basic Surveillance | 5 | The field element shall return sensor and CCTV system fault data to the controlling center for repair. | Roadway Surveillance | 5 | The field element shall return sensor and CCTV system fault data to the controlling center for repair. |
| City of Ocala Field Equipment | Modified | RSE Intersection Management | 4 | The field element shall receive signal priority requests from commercial vehicles and forward to the traffic signal controller. | RSE Intersection Management | 4 | The field element shall receive signal priority/preemption requests from vehicles. |
| City of Ocala Field Equipment | Replaced | Roadway Basic Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. | Roadway Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. |
| City of Ocala Field Equipment | Replaced | Roadway Basic Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. | Roadway Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. |
| City of Ocala Field Equipment | Replaced | Roadway Basic Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. | Roadway Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. |
| City of Ocala Field Equipment | Replaced | Roadway Basic Surveillance | 5 | The field element shall return sensor and CCTV system fault data to the controlling center for repair. | Roadway Surveillance | 5 | The field element shall return sensor and CCTV system fault data to the controlling center for repair. |
| City of Ocala Field Equipment | Replaced | Roadway Basic Surveillance | 6 | The field element shall collect, process, and send traffic images to the center for further wrong way vehicle entries analysis and detection. | Roadway Surveillance | 6 | The field element shall collect, process, and send traffic images to the center for further wrong way vehicle entries analysis and detection. |
| City of Orlando CAV Field Equipment | Modified | RSE Map Management | 10 | The field element shall provide intersection geometry updates to proximate Vehicles. | RSE Map Management | 10 | The field element shall acquire intersection geometry information from mapping centers. |
| City of Orlando CAV Field Equipment | Modified | RSE Map Management | 11 | The field element shall provide parking facility geometry information to proximate Personal devices. | RSE Map Management | 11 | The field element shall acquire parking facility geometry information from mapping centers. |
| City of Orlando CAV Field Equipment | Modified | RSE Map Management | 12 | The field element shall provide roadway geometry updates to proximate Vehicles. | RSE Map Management | 12 | The field element shall acquire roadway geometry information from mapping centers. |
| City of Orlando CAV Field Equipment | Modified | RSE Map Management | 9 | The field element shall provide basemap updates to proximate Vehicles. | RSE Map Management | 9 | The field element shall acquire basemap updates from mapping centers. |
| City of Orlando Field Equipment | Modified | RSE Map Management | 10 | The field element shall provide intersection geometry updates to proximate Vehicles. | RSE Map Management | 10 | The field element shall acquire intersection geometry information from mapping centers. |
| City of Orlando Field Equipment | Replaced | Roadway Basic Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. | Roadway Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. |
| City of Orlando Field Equipment | Replaced | Roadway Basic Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. | Roadway Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. |
| City of Orlando Field Equipment | Replaced | Roadway Basic Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. | Roadway Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. |
| City of Orlando Field Equipment | Replaced | Roadway Basic Surveillance | 5 | The field element shall return sensor and CCTV system fault data to the controlling center for repair. | Roadway Surveillance | 5 | The field element shall return sensor and CCTV system fault data to the controlling center for repair. |
| City of Orlando Field Equipment | Replaced | Roadway Basic Surveillance | 6 | The field element shall collect, process, and send traffic images to the center for further wrong way vehicle entries analysis and detection. | Roadway Surveillance | 6 | The field element shall collect, process, and send traffic images to the center for further wrong way vehicle entries analysis and detection. |
| City of Palm Coast Field Equipment | Replaced | Roadway Basic Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. | Roadway Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. |
| City of Palm Coast Field Equipment | Replaced | Roadway Basic Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. | Roadway Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. |
| City of Palm Coast Field Equipment | Replaced | Roadway Basic Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. | Roadway Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. |
| City of Palm Coast Field Equipment | Replaced | Roadway Basic Surveillance | 5 | The field element shall return sensor and CCTV system fault data to the controlling center for repair. | Roadway Surveillance | 5 | The field element shall return sensor and CCTV system fault data to the controlling center for repair. |
| City of Winter Park Field Equipment | Replaced | Roadway Basic Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. | Roadway Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. |
| City of Winter Park Field Equipment | Replaced | Roadway Basic Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. | Roadway Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. |
| City of Winter Park Field Equipment | Replaced | Roadway Basic Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. | Roadway Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. |
| Commercial Vehicle | Modified | Vehicle Control Automation | 3 | The vehicle shall evaluate the likelihood of a collision between two vehicles or a vehicle and a stationary object, based on the proximity of other objects to the vehicle and the current speed and direction of the vehicle. | Vehicle Control Automation | 3 | The vehicle shall evaluate the likelihood of a collision between two vehicles or a vehicle and another objects, based on the proximity and vectors of other objects relative to the vehicle and the current speed and direction of the vehicle. |
| Commercial Vehicle | Modified | Vehicle Control Warning | 2 | The vehicle shall evaluate the likelihood of a collision between two vehicles or a vehicle and a stationary object to warn the driver, based on the proximity of other objects to the vehicle, roadway characteristics, and the current speed and direction of the vehicle. | Vehicle Control Warning | 2 | The vehicle shall evaluate the likelihood of a collision between two vehicles or a vehicle and another object to warn the driver, based on the proximity and vectors of other objects to the vehicle, roadway characteristics, and the current speed and direction of the vehicle. |
| Commercial Vehicle | Replaced | Vehicle Intersection Warning | 1 | Vehicle shall provide vehicle path information to identify if vehicle is performing an unpermitted movement at an intersection such as a stop sign violation or running a red light. | Vehicle Intersection Movement | 1 | The vehicle shall provide vehicle path information to identify if vehicle is performing an unpermitted movement at an intersection such as a stop sign violation or running a red light. |
| Commercial Vehicle | Replaced | Vehicle Intersection Warning | 2 | Vehicle shall provide data describing the vehicle's location in three dimensions, heading, speed, acceleration, braking status, and size. | Vehicle Intersection Movement | 2 | The vehicle shall provide data describing the vehicle's location in three dimensions, heading, speed, acceleration, braking status, and size. |
| Commercial Vehicle | Replaced | Vehicle Intersection Warning | 3 | The vehicle shall receive intersection signal timing information in order for the vehicle to determine if it will safely cross the intersection given its current location and speed. | Vehicle Intersection Movement | 3 | The vehicle shall receive intersection signal timing information in order for the vehicle to determine if it will safely cross the intersection given its current location and speed. |
| Commercial Vehicle | Replaced | Vehicle Intersection Warning | 4 | The vehicle shall be capable of providing warnings to the driver based upon information received regarding pedestrians, cyclists, and other non-motorized users that are sharing the roadway with the vehicle. | Vehicle Intersection Movement | 4 | The vehicle shall be capable of providing warnings to the driver based upon information received regarding pedestrians, cyclists, and other non-motorized users that are sharing the roadway with the vehicle. |
| Commercial Vehicle | Replaced | Vehicle Intersection Warning | 5 | The vehicle shall receive warning from the infrastructure if an intersection violation appears to be imminent. | Vehicle Intersection Movement | 5 | The vehicle shall receive warning from the infrastructure if an intersection violation appears to be imminent. |
| Commercial Vehicle | Replaced | Vehicle Intersection Warning | 6 | The vehicle shall provide to the driver a warning if it looks like the vehicle will go through the intersection after the signal has turned red. | Vehicle Intersection Movement | 6 | The vehicle shall provide to the driver a warning if it looks like the vehicle will go through the intersection after the signal has turned red. |
| County and City Field Equipment | Replaced | Roadway Basic Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. | Roadway Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. |
| County and City Field Equipment | Replaced | Roadway Basic Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. | Roadway Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. |
| County and City Field Equipment | Replaced | Roadway Basic Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. | Roadway Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. |
| County and City Field Equipment | Replaced | Roadway Basic Surveillance | 5 | The field element shall return sensor and CCTV system fault data to the controlling center for repair. | Roadway Surveillance | 5 | The field element shall return sensor and CCTV system fault data to the controlling center for repair. |
| Disney RCID Field Equipment | Replaced | Roadway Basic Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. | Roadway Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. |
| Disney RCID Field Equipment | Replaced | Roadway Basic Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. | Roadway Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. |
| Disney RCID Field Equipment | Replaced | Roadway Basic Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. | Roadway Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. |
| Disney RCID Field Equipment | Replaced | Roadway Basic Surveillance | 5 | The field element shall return sensor and CCTV system fault data to the controlling center for repair. | Roadway Surveillance | 5 | The field element shall return sensor and CCTV system fault data to the controlling center for repair. |
| FDOT District 5 CAV Field Equipment | Modified | RSE Map Management | 10 | The field element shall provide intersection geometry updates to proximate Vehicles. | RSE Map Management | 10 | The field element shall acquire intersection geometry information from mapping centers. |
| FDOT District 5 CAV Field Equipment | Modified | RSE Map Management | 11 | The field element shall provide parking facility geometry information to proximate Personal devices. | RSE Map Management | 11 | The field element shall acquire parking facility geometry information from mapping centers. |
| FDOT District 5 CAV Field Equipment | Modified | RSE Map Management | 12 | The field element shall provide roadway geometry updates to proximate Vehicles. | RSE Map Management | 12 | The field element shall acquire roadway geometry information from mapping centers. |
| FDOT District 5 CAV Field Equipment | Modified | RSE Map Management | 9 | The field element shall provide basemap updates to proximate Vehicles. | RSE Map Management | 9 | The field element shall acquire basemap updates from mapping centers. |
| FDOT District 5 Field Equipment | Modified | RSE Intersection Management | 4 | The field element shall receive signal priority requests from commercial vehicles and forward to the traffic signal controller. | RSE Intersection Management | 4 | The field element shall receive signal priority/preemption requests from vehicles. |
| FDOT District 5 Field Equipment | Modified | RSE Map Management | 10 | The field element shall provide intersection geometry updates to proximate Vehicles. | RSE Map Management | 10 | The field element shall acquire intersection geometry information from mapping centers. |
| FDOT District 5 Field Equipment | Modified | RSE Map Management | 12 | The field element shall provide roadway geometry updates to proximate Vehicles. | RSE Map Management | 12 | The field element shall acquire roadway geometry information from mapping centers. |
| FDOT District 5 Field Equipment | Modified | RSE Map Management | 9 | The field element shall provide basemap updates to proximate Vehicles. | RSE Map Management | 9 | The field element shall acquire basemap updates from mapping centers. |
| FDOT District 5 Field Equipment | Replaced | Roadway Basic Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. | Roadway Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. |
| FDOT District 5 Field Equipment | Replaced | Roadway Basic Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. | Roadway Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. |
| FDOT District 5 Field Equipment | Replaced | Roadway Basic Surveillance | 3 | The field element shall collect, digitize, and send multimodal crossing and high occupancy vehicle (HOV), and high occupancy toll (HOT) lane sensor data to the center for further analysis and storage. | Roadway Surveillance | 3 | The field element shall collect, digitize, and send multimodal crossing and high occupancy vehicle (HOV), and high occupancy toll (HOT) lane sensor data to the center for further analysis and storage. |
| FDOT District 5 Field Equipment | Replaced | Roadway Basic Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. | Roadway Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. |
| FDOT District 5 Field Equipment | Replaced | Roadway Basic Surveillance | 5 | The field element shall return sensor and CCTV system fault data to the controlling center for repair. | Roadway Surveillance | 5 | The field element shall return sensor and CCTV system fault data to the controlling center for repair. |
| FDOT District 5 Field Equipment | Replaced | Roadway Basic Surveillance | 6 | The field element shall collect, process, and send traffic images to the center for further wrong way vehicle entries analysis and detection. | Roadway Surveillance | 6 | The field element shall collect, process, and send traffic images to the center for further wrong way vehicle entries analysis and detection. |
| FDOT District 5 Smart Work Zone Equipment | Replaced | Roadway Basic Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. | Roadway Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. |
| FDOT District 5 Wildlife Detection System | Replaced | Roadway Basic Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. | Roadway Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. |
| FDOT District 5 Wildlife Detection System | Replaced | Roadway Basic Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. | Roadway Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. |
| FDOT District 5 Wrong-Way Vehicle Detection System (WWVDS) | Replaced | Roadway Basic Surveillance | 6 | The field element shall collect, process, and send traffic images to the center for further wrong way vehicle entries analysis and detection. | Roadway Surveillance | 6 | The field element shall collect, process, and send traffic images to the center for further wrong way vehicle entries analysis and detection. |
| Lake County CAV Field Equipment | Modified | RSE Map Management | 10 | The field element shall provide intersection geometry updates to proximate Vehicles. | RSE Map Management | 10 | The field element shall acquire intersection geometry information from mapping centers. |
| Lake County CAV Field Equipment | Modified | RSE Map Management | 9 | The field element shall provide basemap updates to proximate Vehicles. | RSE Map Management | 9 | The field element shall acquire basemap updates from mapping centers. |
| Lake County Field Equipment | Replaced | Roadway Basic Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. | Roadway Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. |
| Lake County Field Equipment | Replaced | Roadway Basic Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. | Roadway Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. |
| Lake County Field Equipment | Replaced | Roadway Basic Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. | Roadway Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. |
| LYNX CAV Field Equipment | Modified | RSE Map Management | 10 | The field element shall provide intersection geometry updates to proximate Vehicles. | RSE Map Management | 10 | The field element shall acquire intersection geometry information from mapping centers. |
| LYNX CAV Field Equipment | Modified | RSE Map Management | 11 | The field element shall provide parking facility geometry information to proximate Personal devices. | RSE Map Management | 11 | The field element shall acquire parking facility geometry information from mapping centers. |
| LYNX CAV Field Equipment | Modified | RSE Map Management | 12 | The field element shall provide roadway geometry updates to proximate Vehicles. | RSE Map Management | 12 | The field element shall acquire roadway geometry information from mapping centers. |
| LYNX CAV Field Equipment | Modified | RSE Map Management | 9 | The field element shall provide basemap updates to proximate Vehicles. | RSE Map Management | 9 | The field element shall acquire basemap updates from mapping centers. |
| LYNX Transit Vehicles | Replaced | Vehicle Intersection Warning | 1 | Vehicle shall provide vehicle path information to identify if vehicle is performing an unpermitted movement at an intersection such as a stop sign violation or running a red light. | Vehicle Intersection Movement | 1 | The vehicle shall provide vehicle path information to identify if vehicle is performing an unpermitted movement at an intersection such as a stop sign violation or running a red light. |
| Marion County Field Equipment | Replaced | Roadway Basic Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. | Roadway Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. |
| Marion County Field Equipment | Replaced | Roadway Basic Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. | Roadway Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. |
| Marion County Field Equipment | Replaced | Roadway Basic Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. | Roadway Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. |
| Orange County CAV Field Equipment | Modified | RSE Intersection Management | 4 | The field element shall receive signal priority requests from commercial vehicles and forward to the traffic signal controller. | RSE Intersection Management | 4 | The field element shall receive signal priority/preemption requests from vehicles. |
| Orange County CAV Field Equipment | Modified | RSE Map Management | 10 | The field element shall provide intersection geometry updates to proximate Vehicles. | RSE Map Management | 10 | The field element shall acquire intersection geometry information from mapping centers. |
| Orange County CAV Field Equipment | Modified | RSE Map Management | 11 | The field element shall provide parking facility geometry information to proximate Personal devices. | RSE Map Management | 11 | The field element shall acquire parking facility geometry information from mapping centers. |
| Orange County CAV Field Equipment | Modified | RSE Map Management | 12 | The field element shall provide roadway geometry updates to proximate Vehicles. | RSE Map Management | 12 | The field element shall acquire roadway geometry information from mapping centers. |
| Orange County CAV Field Equipment | Modified | RSE Map Management | 9 | The field element shall provide basemap updates to proximate Vehicles. | RSE Map Management | 9 | The field element shall acquire basemap updates from mapping centers. |
| Orange County Field Equipment | Replaced | Roadway Basic Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. | Roadway Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. |
| Orange County Field Equipment | Replaced | Roadway Basic Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. | Roadway Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. |
| Orange County Field Equipment | Replaced | Roadway Basic Surveillance | 3 | The field element shall collect, digitize, and send multimodal crossing and high occupancy vehicle (HOV), and high occupancy toll (HOT) lane sensor data to the center for further analysis and storage. | Roadway Surveillance | 3 | The field element shall collect, digitize, and send multimodal crossing and high occupancy vehicle (HOV), and high occupancy toll (HOT) lane sensor data to the center for further analysis and storage. |
| Orange County Field Equipment | Replaced | Roadway Basic Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. | Roadway Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. |
| Orange County Field Equipment | Replaced | Roadway Basic Surveillance | 6 | The field element shall collect, process, and send traffic images to the center for further wrong way vehicle entries analysis and detection. | Roadway Surveillance | 6 | The field element shall collect, process, and send traffic images to the center for further wrong way vehicle entries analysis and detection. |
| Osceola County CAV Field Equipment | Modified | RSE Map Management | 10 | The field element shall provide intersection geometry updates to proximate Vehicles. | RSE Map Management | 10 | The field element shall acquire intersection geometry information from mapping centers. |
| Osceola County CAV Field Equipment | Modified | RSE Map Management | 11 | The field element shall provide parking facility geometry information to proximate Personal devices. | RSE Map Management | 11 | The field element shall acquire parking facility geometry information from mapping centers. |
| Osceola County CAV Field Equipment | Modified | RSE Map Management | 12 | The field element shall provide roadway geometry updates to proximate Vehicles. | RSE Map Management | 12 | The field element shall acquire roadway geometry information from mapping centers. |
| Osceola County CAV Field Equipment | Modified | RSE Map Management | 9 | The field element shall provide basemap updates to proximate Vehicles. | RSE Map Management | 9 | The field element shall acquire basemap updates from mapping centers. |
| Osceola County Field Equipment | Replaced | Roadway Basic Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. | Roadway Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. |
| Osceola County Field Equipment | Replaced | Roadway Basic Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. | Roadway Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. |
| Osceola County Field Equipment | Replaced | Roadway Basic Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. | Roadway Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. |
| Osceola County Field Equipment | Replaced | Roadway Basic Surveillance | 6 | The field element shall collect, process, and send traffic images to the center for further wrong way vehicle entries analysis and detection. | Roadway Surveillance | 6 | The field element shall collect, process, and send traffic images to the center for further wrong way vehicle entries analysis and detection. |
| Other Vehicle | Modified | Vehicle Control Warning | 2 | The vehicle shall evaluate the likelihood of a collision between two vehicles or a vehicle and a stationary object to warn the driver, based on the proximity of other objects to the vehicle, roadway characteristics, and the current speed and direction of the vehicle. | Vehicle Control Warning | 2 | The vehicle shall evaluate the likelihood of a collision between two vehicles or a vehicle and another object to warn the driver, based on the proximity and vectors of other objects to the vehicle, roadway characteristics, and the current speed and direction of the vehicle. |
| Private Travelers Personal Computing Devices | Modified | Personal Trip Planning and Route Guidance | 12 | The personal traveler interface to identify trip planning parameters: Origin, Destination, departure time, arrival time, acceptable modes, so that they might plan for a single coordinated trip using multiple modes using a single action. | Personal Trip Planning and Route Guidance | 12 | The personal traveler interface shall provide means for the Traveler to identify trip planning parameters including origin, destination, departure time, arrival time, acceptable modes, so that they might plan for a single coordinated trip using multiple modes in a single action. |
| Seminole County CAV Field Equipment | Modified | RSE Map Management | 10 | The field element shall provide intersection geometry updates to proximate Vehicles. | RSE Map Management | 10 | The field element shall acquire intersection geometry information from mapping centers. |
| Seminole County CAV Field Equipment | Modified | RSE Map Management | 11 | The field element shall provide parking facility geometry information to proximate Personal devices. | RSE Map Management | 11 | The field element shall acquire parking facility geometry information from mapping centers. |
| Seminole County CAV Field Equipment | Modified | RSE Map Management | 12 | The field element shall provide roadway geometry updates to proximate Vehicles. | RSE Map Management | 12 | The field element shall acquire roadway geometry information from mapping centers. |
| Seminole County CAV Field Equipment | Modified | RSE Map Management | 9 | The field element shall provide basemap updates to proximate Vehicles. | RSE Map Management | 9 | The field element shall acquire basemap updates from mapping centers. |
| Seminole County Field Equipment | Replaced | Roadway Basic Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. | Roadway Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. |
| Seminole County Field Equipment | Replaced | Roadway Basic Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. | Roadway Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. |
| Seminole County Field Equipment | Replaced | Roadway Basic Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. | Roadway Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. |
| Seminole County Field Equipment | Replaced | Roadway Basic Surveillance | 5 | The field element shall return sensor and CCTV system fault data to the controlling center for repair. | Roadway Surveillance | 5 | The field element shall return sensor and CCTV system fault data to the controlling center for repair. |
| Sumter County Field Equipment | Replaced | Roadway Basic Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. | Roadway Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. |
| Sumter County Field Equipment | Replaced | Roadway Basic Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. | Roadway Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. |
| Sumter County Field Equipment | Replaced | Roadway Basic Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. | Roadway Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. |
| Vehicle | Modified | Vehicle Control Automation | 3 | The vehicle shall evaluate the likelihood of a collision between two vehicles or a vehicle and a stationary object, based on the proximity of other objects to the vehicle and the current speed and direction of the vehicle. | Vehicle Control Automation | 3 | The vehicle shall evaluate the likelihood of a collision between two vehicles or a vehicle and another objects, based on the proximity and vectors of other objects relative to the vehicle and the current speed and direction of the vehicle. |
| Vehicle | Modified | Vehicle Control Warning | 2 | The vehicle shall evaluate the likelihood of a collision between two vehicles or a vehicle and a stationary object to warn the driver, based on the proximity of other objects to the vehicle, roadway characteristics, and the current speed and direction of the vehicle. | Vehicle Control Warning | 2 | The vehicle shall evaluate the likelihood of a collision between two vehicles or a vehicle and another object to warn the driver, based on the proximity and vectors of other objects to the vehicle, roadway characteristics, and the current speed and direction of the vehicle. |
| Vehicle | Replaced | Vehicle Intersection Warning | 1 | Vehicle shall provide vehicle path information to identify if vehicle is performing an unpermitted movement at an intersection such as a stop sign violation or running a red light. | Vehicle Intersection Movement | 1 | The vehicle shall provide vehicle path information to identify if vehicle is performing an unpermitted movement at an intersection such as a stop sign violation or running a red light. |
| Vehicle | Replaced | Vehicle Intersection Warning | 2 | Vehicle shall provide data describing the vehicle's location in three dimensions, heading, speed, acceleration, braking status, and size. | Vehicle Intersection Movement | 2 | The vehicle shall provide data describing the vehicle's location in three dimensions, heading, speed, acceleration, braking status, and size. |
| Vehicle | Replaced | Vehicle Intersection Warning | 3 | The vehicle shall receive intersection signal timing information in order for the vehicle to determine if it will safely cross the intersection given its current location and speed. | Vehicle Intersection Movement | 3 | The vehicle shall receive intersection signal timing information in order for the vehicle to determine if it will safely cross the intersection given its current location and speed. |
| Vehicle | Replaced | Vehicle Intersection Warning | 4 | The vehicle shall be capable of providing warnings to the driver based upon information received regarding pedestrians, cyclists, and other non-motorized users that are sharing the roadway with the vehicle. | Vehicle Intersection Movement | 4 | The vehicle shall be capable of providing warnings to the driver based upon information received regarding pedestrians, cyclists, and other non-motorized users that are sharing the roadway with the vehicle. |
| Vehicle | Replaced | Vehicle Intersection Warning | 5 | The vehicle shall receive warning from the infrastructure if an intersection violation appears to be imminent. | Vehicle Intersection Movement | 5 | The vehicle shall receive warning from the infrastructure if an intersection violation appears to be imminent. |
| Vehicle | Replaced | Vehicle Intersection Warning | 6 | The vehicle shall provide to the driver a warning if it looks like the vehicle will go through the intersection after the signal has turned red. | Vehicle Intersection Movement | 6 | The vehicle shall provide to the driver a warning if it looks like the vehicle will go through the intersection after the signal has turned red. |
| Volusia County Field Equipment | Replaced | Roadway Basic Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. | Roadway Surveillance | 1 | The field element shall collect, process, digitize, and send traffic sensor data (speed, volume, and occupancy) to the center for further analysis and storage, under center control. |
| Volusia County Field Equipment | Replaced | Roadway Basic Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. | Roadway Surveillance | 2 | The field element shall collect, process, and send traffic images to the center for further analysis and distribution. |
| Volusia County Field Equipment | Replaced | Roadway Basic Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. | Roadway Surveillance | 4 | The field element shall return sensor and CCTV system operational status to the controlling center. |